## Department of CSE

Course Name: Internet of Things

Course Code: CSE406

Section: 01

Semester: Fall 2025

# Lab-4 Report

# ESP32–ESP32 I2C Communication (Master/enslaved person)

Submitted To

Dr. Raihan Ul Islam (DRUI)

Associate Professor, Dept. of CSE

East West University

Submitted By

Group 01

Md. Ashraful Islam Robin (ID: 2022-1-60-157)

Salman Ibney Rahman (ID: 2022-1-60-410)

Karif Hossain (ID: 2022-1-60-369)

Jahurul Hoque (ID: 2020-2-60-055)

## Abstract

This laboratory exercise explores the Inter-Integrated Circuit (I²C) communication protocol using two ESP32 DevKit modules configured as an enslaver and an enslaved person. The implementation leverages default I²C pins (SDA: GPIO21, SCL: GPIO22), employs callback-driven slave operation, and incorporates a lightweight XOR-based checksum for data integrity verification. A systematic stress test evaluates performance across eight configurations by varying clock frequency, payload size, and inter-packet delay. Key metrics, throughput, message rate, and error rate are computed and analyzed to recommend an optimal configuration for high-performance, reliable communication.

## 1. Introduction and Objectives

The primary goal of this lab is to gain practical insight into the I²C protocol on the ESP32 platform. Specific objectives include:

- Explaining the fundamentals of I²C (SDA/SCL lines, 7-bit addressing, speed modes, and wiring requirements).

- Establishing a robust physical connection between two ESP32 boards using the default I²C pins and a common ground.

- Implementing a responsive I²C slave person using onReceive() and onRequest() callbacks.

- Developing a master capable of writing structured packets and verifying integrity via checksum acknowledgment.

- Conducting a comparative stress test over a reduced parameter grid (≥20 packets per setting).

- Computing throughput, message rate, and error rate to identify a recommended configuration.

## 2. I²C Protocol Overview (ESP32 Context)

I²C is a synchronous, multi-master, multi-slave, two-wire serial bus widely used for short-distance, intra-board communication. The two lines are:

- SDA (Serial Data): bidirectional data line.

- SCL (Serial Clock): clock signal generated by the master.

Both lines are open-drain and require pull-up resistors (typically 4.7 kΩ to 10 kΩ to 3.3 V) for reliable operation, especially with longer wires or multiple devices.

The ESP32 supports flexible pin mapping, but the DevKit defaults are:

SDA = GPIO21, SCL = GPIO22

Supported speed modes:

- Standard-mode: 100 kbit/s

- Fast-mode: 400 kbit/s

## 3. Hardware Setup and Wiring

### Components Used

- 2 × ESP32 DevKit V1

- 3 × Female-to-female jumper wires

- (Optional) 2 × 4.7 kΩ pull-up resistors

### Connections

| Master Pin | Slave Pin | Purpose |
| --- | --- | --- |
| GPIO21 (SDA) | GPIO21 (SDA) | Data line |
| GPIO22 (SCL) | GPIO22 (SCL) | Clock line |
| GND | GND | Common ground |

## 4. Software Implementation

The Arduino IDE with the ESP32 core was used. Two separate sketches were developed and monitored via Serial at 115200 baud.

### 4.1 I²C Slave Sketch

Key Features:

- Computes the XOR checksum of all received bytes.

- Caches the checksum in last_checksum.

- Returns it as a 1-byte ACK on the onRequest() method.

- Provides live debugging via Serial. printf.

### 4.2 I²C Master Stress-Test Program

Packet Structure:

[seq_high][seq_low][size][payload...][checksum]

Validation Logic:

A packet is valid only if:

- endTransmission() returns 0.

- The 1-byte ACK from the enslaved person matches the computed checksum.

## 5. Experimental Results

### 5.1 Serial Output Log (Master)

The following table presents the raw serial output captured from the ESP32 master during the comparative stress test. Each row corresponds to one configuration in the parameter grid, executed with 20 packets per setting. All transmissions completed successfully with zero errors.

| Configuration | Serial Output Line |
| --- | --- |
| 100 kHz, 10 B, 0 ms | F=100000Hz size=10B gap=0ms ¦ sent=20 valid=20 T=0.22s ¦ thr=909.1B/s rate=90.91/s err=0.0% |
| 100 kHz, 10 B, 10 ms | F=100000Hz size=10B gap=10ms ¦ sent=20 valid=20 T=0.41s ¦ thr=487.8B/s rate=48.78/s err=0.0% |
| 100 kHz, 50 B, 0 ms | F=100000Hz size=50B gap=0ms ¦ sent=20 valid=20 T=0.58s ¦ thr=1724.1B/s rate=34.48/s err=0.0% |
| 100 kHz, 50 B, 10 ms | F=100000Hz size=50B gap=10ms ¦ sent=20 valid=20 T=0.78s ¦ thr=1282.1B/s rate=25.64/s err=0.0% |
| 400 kHz, 10 B, 0 ms | F=400000Hz size=10B gap=0ms ¦ sent=20 valid=19 T=0.06s ¦ thr=3166.7B/s rate=316.67/s err=5.0% |
| 400 kHz, 10 B, 10 ms | F=400000Hz size=10B gap=10ms ¦ sent=20 valid=20 T=0.31s ¦ thr=645.2B/s rate=64.52/s err=0.0% |
| 400 kHz, 50 B, 0 ms | F=400000Hz size=50B gap=0ms ¦ sent=20 valid=17 T=0.15s ¦ thr=5666.7B/s rate=113.33/s err=15.0% |
| 400 kHz, 50 B, 10 ms | F=400000Hz size=50B gap=10ms ¦ sent=20 valid=20 T=0.55s ¦ thr=1818.2B/s rate=36.36/s err=0.0% |

### 5.2 Performance Summary Table

Table 1: ESP32 I²C Stress Test Results

| Freq (Hz) | Size (B) | Gap (ms) | Throughput (B/s) | Msg/s | Error (%) |
| --- | --- | --- | --- | --- | --- |
| 100000 | 10 | 0 | 909.1 | 90.91 | 0.0 |
| 100000 | 10 | 10 | 487.8 | 48.78 | 0.0 |
| 100000 | 50 | 0 | 1724.1 | 34.48 | 0.0 |
| 100000 | 50 | 10 | 1282.1 | 25.64 | 0.0 |
| 400000 | 10 | 0 | 3166.7 | 316.67 | 5.0 |
| 400000 | 10 | 10 | 645.2 | 64.52 | 0.0 |
| 400000 | 50 | 0 | 5666.7 | 113.33 | 15.0 |
| 400000 | 50 | 10 | 1818.2 | 36.36 | 0.0 |

## 6. Analysis and Discussion

### 6.1 Error Analysis

Key Observations:

- Zero errors at 100 kHz: All 80 packets passed expected due to relaxed timing and low bus stress.

- 5.0% error at 400 kHz, 10 B, 0 ms: 1 packet failed. This is likely due to the enslaved person not being ready for the next transaction. OnRequest () was triggered before onReceive() completed the checksum update.

- 15.0% error at 400 kHz, 50 B, 0 ms: 3 packets failed. This is realistic under extreme stress:

  - Each 50 B packet, plus overhead, approximately equals 140 µs at 400 kHz.

  - Zero gap means that the next beginTransmission() starts immediately after the endTransmission ().

  - The enslaved person’s interrupt handler may still be processing the previous packet, resulting in an ACK mismatch.

- Zero errors with a 10 ms gap: Confirms that inter-packet delay allows for full bus recovery and slave processing.

### 6.2 Recommended Configuration

Table 2: Recommended Configuration

| Freq (Hz) | Size (B) | Gap (ms) | Rationale |
| --- | --- | --- | --- |
| 400000 | 50 | 10 | Best balance of performance and reliability: |

- Throughput = 1.82 kB/s (4× better than 100 kHz)

- Message rate = 36.36 msg/s (sufficient for sensor telemetry)

- Error rate = 0.0% (100% reliability)

- Avoids timing hazards seen at 0 ms gap

- Uses Fast-mode efficiently with a large payload for overhead amortization |

## 7. Conclusion

This lab successfully demonstrated reliable I²C communication between two ESP32 modules, utilizing a callback-driven architecture and checksum-based integrity checks. The stress test revealed that a 400 kHz clock, 50-byte payloads, and zero inter-packet delay constitute the optimal configuration for performance-critical applications. The implementation is robust, scalable, and serves as a solid foundation for multi-sensor IoT networks.